MAINCRAFTS TECHNOLOGY

VLSI DESIGN INTERNSHIP

Task 3 Completion Report

RTL Design & Verification of Sequential Circuits and Flip-Flops

Verilog HDL Implementation of D Flip-Flop, JK Flip-Flop, 4-bit Register, and 4-bit Counter

| Intern Name | Bathula Thrishul |
| --- | --- |
| Domain | VLSI Design |
| Platform Used | EDA Playground (Icarus Verilog 12.0 + EPWave) |
| Submission Date | August 20, 2026 |

# 1. Executive Objective

Building on Task 1 (VLSI fundamentals) and Task 2 (combinational RTL design), Task 3 moves from combinational to sequential logic — the foundation of processors, controllers, and memory systems. This report documents the Verilog RTL implementation, testbench creation, console simulation, and EPWave waveform verification for four sequential building blocks:

- D Flip-Flop — edge-triggered data storage element

- JK Flip-Flop — versatile flip-flop with Set / Reset / Toggle / No-change modes

- 4-bit Register — parallel storage of multi-bit data

- 4-bit Binary Counter — clock-driven up-counter with roll-over

All four designs were written, compiled, and simulated on EDA Playground using Icarus Verilog 12.0, with waveforms captured in EPWave to confirm correct clock-synchronous behavior.

# 2. Sequential RTL Design Concepts

| Concept | Explanation |
| --- | --- |
| Sequential Logic | Circuit behavior that depends on current inputs AND previous state (memory). |
| Flip-Flop | The basic 1-bit storage element, updated only at a clock edge. |
| Clock Signal | Synchronizes when state updates occur across the circuit. |
| Register | A group of flip-flops that together store multi-bit data. |
| Counter | A sequential circuit that increments its stored value every clock cycle. |
| always @(posedge clk) | The Verilog construct used to describe clock-edge-triggered sequential behavior. |

# 3. Circuit Implementations, Simulation & Results

## Part A — D Flip-Flop

1. Truth Table

| Clock Edge | D | Q (Next State) |
| --- | --- | --- |
| Rising Edge | 0 | 0 |
| Rising Edge | 1 | 1 |

2. Verilog RTL Design (d_flipflop.v)

module d_flipflop( input clk, input D, output reg Q ); always @(posedge clk) begin Q <= D; end endmodule

3. Verification Testbench (tb_d_flipflop.v) — EDA Playground Setup

Figure A1 — EDA Playground: d_flipflop.v design and testbench (clock period 10 ns, D toggled every 10 ns)

4. Simulation Log

[2026-08-20 13:13:01 UTC] iverilog -Wall -g2012 design.sv testbench.sv && vvp a.out VCD info: dumpfile dump.vcd opened for output. testbench.sv:34: $finish called at 50000 (1ps) Finding VCD file... ./dump.vcd Opening EPWave... Done

5. EPWave Waveform

Figure A2 — clk, D and Q signal transitions (0–35,000 ps): Q follows D exactly one clock edge later

Observation: The output Q updates only on the rising edge of clk and holds its value between edges, confirming correct edge-triggered, non-blocking (<=) sequential behavior with no glitches or unintended combinational feed-through.

## Part B — JK Flip-Flop

1. Truth Table

| J | K | Q (Next State) |
| --- | --- | --- |
| 0 | 0 | No change |
| 0 | 1 | Reset |
| 1 | 0 | Set |
| 1 | 1 | Toggle |

2. Verilog RTL Design (jk_flipflop.v)

module jk_flipflop( input clk, input J, input K, output reg Q ); always @(posedge clk) begin case ({J, K}) 2'b00: Q <= Q; // No change 2'b01: Q <= 1'b0; // Reset 2'b10: Q <= 1'b1; // Set 2'b11: Q <= ~Q; // Toggle endcase end endmodule

3. Verification Testbench — EDA Playground Setup

Figure B1 — EDA Playground: jk_flipflop.v design and testbench, exercising all four J/K combinations

4. Simulation Log

[2026-08-20 13:15:00 UTC] iverilog -Wall -g2012 design.sv testbench.sv && vvp a.out VCD info: dumpfile dump.vcd opened for output. testbench.sv:43: $finish called at 90000 (1ps) Finding VCD file... ./dump.vcd Opening EPWave... Done

5. EPWave Waveform

Figure B2 — clk, J, K and Q transitions (0–70,000 ps) across No-change, Reset, Set and Toggle phases

Observation: Q correctly held its previous value for J=K=0, reset to 0 for J=0,K=1, set to 1 for J=1,K=0, and toggled every clock edge for J=K=1 — matching the JK flip-flop truth table exactly.

## Part C — 4-bit Register

Registers store multiple bits of data, updated together on the same clock edge.

1. Verilog RTL Design (register4.v)

module register4( input clk, input [3:0] D, output reg [3:0] Q ); always @(posedge clk) begin Q <= D; end endmodule

2. Verification Testbench — EDA Playground Setup

Figure C1 — EDA Playground: register4.v design and testbench (D driven with 4'b1010, 4'b0101, 4'b1111, 4'b0011)

3. Simulation Log

[2026-08-20 13:16:31 UTC] iverilog -Wall -g2012 design.sv testbench.sv && vvp a.out VCD info: dumpfile dump.vcd opened for output. testbench.sv:29: $finish called at 60000 (1ps) Finding VCD file... ./dump.vcd Opening EPWave... Done

4. EPWave Waveform

Figure C2 — D[3:0] and Q[3:0] (0–45,000 ps): Q follows D one clock edge later, values 0 → a → 5 → f → 3 (hex)

Observation: All four bits of Q updated simultaneously and only at the rising clock edge, confirming the 4-bit register behaves as four parallel D flip-flops sharing a common clock.

## Part D — 4-bit Binary Counter

A counter increments its stored value on every clock cycle and rolls over after reaching its maximum count.

1. Verilog RTL Design (counter4.v)

module counter4( input clk, output reg [3:0] count ); initial count = 4'b0000; // Safe initial state for simulation always @(posedge clk) begin count <= count + 1'b1; end endmodule

2. Verification Testbench — EDA Playground Setup

Figure D1 — EDA Playground: counter4.v design and testbench, run for 200 ns to observe a full 0–15–0 roll-over

3. Simulation Log

[2026-08-20 13:18:31 UTC] iverilog -Wall -g2012 design.sv testbench.sv && vvp a.out VCD info: dumpfile dump.vcd opened for output. testbench.sv:23: $finish called at 200000 (1ps) Finding VCD file... ./dump.vcd Opening EPWave... Done

4. EPWave Waveform

Figure D2 — count[3:0] incrementing every clock cycle (0–190,000 ps), rolling over cleanly from f back to 0

Observation: count increased by exactly 1 on every rising clock edge and wrapped from 4'hF back to 4'h0 without glitches, confirming correct 4-bit modulo-16 counting behavior.

# 4. Key Takeaways & Conclusion

- Functional Correctness: All four sequential modules used non-blocking assignments (<=) inside always @(posedge clk) blocks, correctly modeling clocked storage with no unintended latches or combinational feedback.

- State Behavior Verified: D flip-flop, JK flip-flop (all 4 modes), register, and counter all matched their respective truth tables / expected count sequences across every EPWave waveform capture.

- Simulation Workflow: Successfully used $dumpfile and $dumpvars in each testbench to generate VCD files for waveform inspection in EPWave, mirroring the industry RTL verification flow.

- Task Completion: RTL modules, testbenches, console logs, and waveform diagrams satisfy all mandatory deliverables for Task 3 of the VLSI Design Internship.

Bathula Thrishul  ·  VLSI Design Intern, Maincrafts Technology  ·  August 2026